Turbomachinery design often starts with the specific speed, defined as:

Ns=qv,in∆hs34 ,			(1)

where Ω [rad/s] is the angular velocity, qv,in [m3/s] is the inlet volumetric flow rate, and Δhs [J/kg] is the required isentropic enthalpy difference. The specific speed can be used to design any type of turbomachine – axial and radial compressors, axial and radial turbines, pumps and fans. Inlet volume flow is based on the required mass flow rate of the machine, and the inlet thermodynamic state, and the required isentropic enthalpy difference depends on the inlet state and the required pressure ratio, and the working fluid itself. For a fixed pressure ratio, the lower the molar mass of the fluid, the higher is the isentropic enthalpy difference. For example, the required isentropic enthalpy difference in a hydrogen compressor is around 14.5 times as high as for an air compressor, given a similar pressure ratio.

For each specific machine type, there is a certain specific speed range, which leads to a good isentropic efficiency. For example, for centrifugal compressors the good range is between 0.6 and 0.9 (Rodgers, 1980). Once the designer has chosen the design specific speed, the corresponding rotational speed can be solved. It should be noted that the specific speed ranges are not absolute, rather they are semi-empirical general guidelines. In some cases, it may be viable to design the machine at sub-optimal specific speed sacrificing some of the efficiency.

REFERENCES:

Rodgers, C. (1980), Specific speed and efficiency of centrifugal impellers, The 25th annual international gas turbine conference and exhibit and the 22nd annual fluids engineering conference, March 9-13, New Orleans, Louisiana, USA