Turbomachinery design often starts with the specific speed, defined as:

Ns=qv,in∆hs34 ,			(1)

where Ω [rad/s] is the angular velocity, qv,in [m3/s] is the inlet volumetric flow rate, and Δhs [J/kg] is the required isentropic enthalpy difference. The specific speed can be used to design any type of turbomachine – axial and radial compressors, axial and radial turbines, pumps and fans. Inlet volume flow is based on the required mass flow rate of the machine, and the inlet thermodynamic state, and the required isentropic enthalpy difference depends on the inlet state and the required pressure ratio, and the working fluid itself. For a fixed pressure ratio, the lower the molar mass of the fluid, the higher is the isentropic enthalpy difference. For example, the required isentropic enthalpy difference in a hydrogen compressor is around 14.5 times as high as for an air compressor, given a similar pressure ratio. For an ideal gas, the required enthalpy increase is:

∆hs=Ri1-1γTout,s-Tin,			(2)

where Ri [J/kgK] is the specific gas constant, γ is the ratio of specific heat capacities, Tout,s [K] is the isentropic compressor outlet temperature, and Tin [K] is the compressor inlet temperature. The specific gas constant is:

Ri=RiM,				(3)

where Ru [J/molK] is the universal gas constant and M [kg/mol] is the molar mass of the working fluid. Furthermore, for an ideal gas, the isentropic outlet temperature is: 	Tout,s=Tin(PR)γ-1γ,			(4)

where PR is the compressor pressure ratio. Additionally, the inlet volume flow is:

qv,in= qmρin,				(5)

where qm [kg/s] is the desired mass flow and ρin [kg/m3] is the inlet density of the working fluid. For a perfect gas, the density can be obtained from the ideal gas equation of state:

ρin=pinRiTin,				(6)

where pin s the compressor inlet pressure. For non-ideal working fluids, the thermodynamic fluid properties must be obtained with thermodynamic libraries, such as CoolProp or Refprop.

For each specific machine type, there is a certain specific speed range, which leads to a good isentropic efficiency. For example, for centrifugal compressors the good range is between 0.6 and 0.9 (Rodgers, 1980). Once the designer has chosen the design specific speed, the corresponding rotational speed can be solved. It should be noted that the specific speed ranges are not absolute, rather they are semi-empirical general guidelines. In some cases, it may be viable to design the machine at sub-optimal specific speed sacrificing some of the efficiency. The use of the specific speed to assess the desired compressor rotational speed is exemplified in Table 1, where the rotational speed for two compressors – one for air and one for hydrogen is estimated. Both fluids are considered perfect, both have the same inlet pressure and temperature, 1 bar and 15°C, the same mass flow, 3 kg/s, and the same pressure ratio of 3.

Table 1. Desired compressor rotational speeds. Both compressors have the same inlet conditions: pressure 1 bar, temperature 15°C, and mass flow 3 kg/s, and the pressure ratio for both is 3.

| Machine | Fluid | Ns | pin | M | Ri | γ | ρin | qv,in | Δhs | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | [-] | [bar] | [kg/mol] | [J/kgK] | [-] | [g/m^3] | [m^3/s] | [J/kg] | [rpm] |
| Compressor | Air | 0.80 | 1 | 0.0290 | 287 | 1.40 | 1.277 | 2.3497 | 106615 | 29405 |
| Compressor | Hydrogen | 0.80 | 1 | 0.0020 | 4157 | 1.41 | 0.088 | 34.0650 | 1549090 | 57473 |

Furthermore, equation (1) show that, for a given working fluid and inlet condition and fixed specific speed, the larger the required mass flow rate is, the lower is the compressor rotational speed, and the higher the required pressure ratio, the higher the rotational speed.

REFERENCES:

Rodgers, C. (1980), Specific speed and efficiency of centrifugal impellers, The 25th annual international gas turbine conference and exhibit and the 22nd annual fluids engineering conference, March 9-13, New Orleans, Louisiana, USA